# Environmental Monitoring Equipment Technical Specifications

Reference: Circular No. 10/2021/TT-BTNMT dated June 30, 2021 (Vietnam)

____________________

# Table 1. Automatic Surface Water Monitoring

| No. | Parameter | Unit | Acc (% rd) | Acc (% FS) | Range | Res | Resp |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Temperature | °C | ± 5% | ± 3% | 0 ÷ 80°C | 0.1 | ≤ 5 s |
| 2 | pH | - | ± 0.1 | ± 0.1 | 0 ÷ 14 | 0.1 | ≤ 5 s |
| 3 | Total Suspended Solids (TSS) | mg/L | ± 5% | ± 2% | 0 ÷ 500 | 0.1 | ≤ 10 s |
| 4 | Chemical Oxygen Demand (COD) | mg/L | ± 5% | ± 5% | 0 ÷ 100 | 0.5 | ≤ 15 m |
| 5 | Dissolved Oxygen (DO) | mg/L | ± 5% | ± 5% | 0 ÷ 20 | 0.1 | ≤ 120 s |
| 6 | Nitrate (NO₃⁻) | mg/L | ± 5% | ± 3% | 0 ÷ 50 | 0.5 | ≤ 10 m |
| 7 | Phosphate (PO₄³⁻) | mg/L | ± 5% | ± 5% | 0 ÷ 2 | - | ≤ 10 m |
| 8 | Ammonium (NH₄⁺) | mg/L | ± 5% | ± 5% | 0 ÷ 5 | 0.2 | ≤ 30 m |
| 9 | Total Phosphorus (TP) | mg/L | ± 5% | ± 3% | 0 ÷ 2 | 0.1 | ≤ 30 m |
| 10 | Total Nitrogen (TN) | mg/L | ± 5% | ± 3% | 0 ÷ 20 | 0.1 | ≤ 30 m |
| 11 | Total Organic Carbon (TOC) | mg/L | ± 5% | ± 2% | 0 ÷ 100 | 0.1 | ≤ 30 m |

# Table 2. Ambient Air Quality Monitoring

| No. | Parameter | Unit | Acc (% rd) | Acc (% FS) | Range | Res | Resp |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Temperature | °C | ± 5% | ± 5% | 0 ÷ 80°C | 0.1 | ≤ 120 s |
| 2 | Nitrogen Dioxide (NO₂) | µg/Nm³ | ± 5% | ± 5% | 0 ÷ 500 | 0.1 | ≤ 300 s |
|  |  | ppb | ± 5% | ± 5% | 0 ÷ 250 | 0.1 |  |
| 3 | Carbon Monoxide (CO) | µg/Nm³ | ± 5% | ± 5% | 0 ÷ 100k | 0.1 | ≤ 200 s |
|  |  | ppb | ± 5% | ± 5% | 0 ÷ 85k | 0.1 |  |
| 4 | Sulfur Dioxide (SO₂) | µg/Nm³ | ± 5% | ± 5% | 0 ÷ 1k | 0.1 | ≤ 200 s |
|  |  | ppb | ± 5% | ± 5% | 0 ÷ 400 | 0.1 |  |
| 5 | Ozone (O₃) | µg/Nm³ | ± 5% | ± 5% | 0 ÷ 500 | 0.1 | ≤ 200 s |
|  |  | ppb | ± 5% | ± 5% | 0 ÷ 250 | 0.1 |  |
| 6 | PM10 | µg/Nm³ | ± 5% | ± 3% | 0 ÷ 500 | 0.1 | ≤ 60 s |
| 7 | PM2.5 | µg/Nm³ | ± 5% | ± 3% | 0 ÷ 150 | 0.1 | ≤ 60 s |

# Table 3. Automatic Wastewater Monitoring

| No. | Parameter | Unit | Acc(rd) | Acc(FS) | Res | Resp |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Flow Rate | m³/h | ± 5% | ± 5% | - | ≤ 5 m |
| 2 | Temperature | °C | ± 5% | ± 5% | 0.1 | ≤ 5 s |
| 3 | Color | Pt-Co | ± 5% | ± 5% | - | ≤ 5 m |
| 4 | pH | - | ± 0.2 | ± 0.2 | 0.1 | ≤ 5 s |
| 5 | TSS | mg/L | ± 5% | ± 3% | 0.1 | ≤ 10 s |
| 6 | COD | mg/L | ± 5% | ± 3% | 0.5 | ≤ 15 m |
| 7 | Ammonium (NH₄⁺) | mg/L | ± 5% | ± 5% | 0.2 | ≤ 30 m |
| 8 | Total Phos (TP) | mg/L | ± 5% | ± 3% | 0.1 | ≤ 30 m |
| 9 | Total Nitro (TN) | mg/L | ± 5% | ± 3% | 0.1 | ≤ 30 m |
| 10 | TOC | mg/L | ± 5% | ± 5% | 0.1 | ≤ 30 m |
| 11 | Res Chlorine | mg/L | ± 5% | ± 2% | 0.1 | ≤ 30 m |

# Table 5. Automatic Stack Emission Monitoring

| No. | Parameter | Unit | Acc(rd) | Acc(FS) | Res | Resp |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Temperature | °C | ± 5% | ± 5% | 0.1 | ≤ 120 s |
| 2 | Pressure | kPa | ± 5% | ± 5% | - | ≤ 120 s |
| 3 | Nitrogen Monoxide (NO) | mg/m³ | ± 5% | ± 5% | 1 | ≤ 200 s |
|  |  | ppm | ± 5% | ± 5% | 1 |  |
| 4 | Nitrogen Dioxide (NO₂) | mg/m³ | ± 5% | ± 5% | 1 | ≤ 300 s |
|  |  | ppm | ± 5% | ± 5% | 1 |  |
| 5 | Carbon Monoxide (CO) | mg/m³ | ± 5% | ± 5% | 1 | ≤ 200 s |
|  |  | ppm | ± 5% | ± 5% | 1 |  |
| 6 | Sulfur Dioxide (SO₂) | mg/m³ | ± 5% | ± 5% | 1 | ≤ 200 s |
|  |  | ppm | ± 5% | ± 5% | 1 |  |
| 7 | Oxygen (O₂) | %V | ± 0.5% | ± 0.5% | 0.1 | ≤ 200 s |
| 8 | Hydrogen Sulfide (H₂S) | mg/m³ | ± 5% | ± 5% | 0.1 | ≤ 300 s |
|  |  | ppm | ± 5% | ± 5% | 0.1 |  |
| 9 | Ammonia (NH₃) | mg/m³ | ± 5% | ± 5% | 0.1 | ≤ 300 s |
|  |  | ppm | ± 5% | ± 5% | 0.1 |  |
| 10 | Mercury Vapor (Hg) | mg/m³ | ± 5% | ± 5% | 0.1 | ≤ 900 s |
| 11 | Particulate Matter (PM) | mg/m³ | ± 10% | ± 5% | 0.1 | ≤ 60 s |